DIY 2-Wheeled Robot

Multiple configurations using BO motors, L293D, HC-05 Bluetooth, Li-ion cells, servo, ultrasonic, IR, RF, and breadboard modules

Practical scope This document gives build-ready configurations for one compact 2-wheeled rover platform. It is written for classroom, prototype, and hobby workshop use, with clear power, wiring, test, and troubleshooting notes.

| Included item | Used for |
| --- | --- |
| 2 x BO gear motor | Left and right drive motors for differential steering. |
| 1 x L293D motor driver | Bidirectional motor control for both BO motors. |
| 1 x Bluetooth HC-05 | Phone/app serial remote control. |
| 2 x 3.7 V Li-ion cells | Nominal 7.4 V battery pack for motor supply. |
| 1 x Micro servo | Ultrasonic scanner or small steering/sensor mount. |
| 1 x Ultrasonic sensor | Front distance detection for obstacle logic. |
| 2 x IR sensors | Line following, edge detection, or near-object triggers. |
| 1 x RF module | Wireless command receiver or telemetry experiment. |
| 1 x Small breadboard | Temporary wiring and power distribution. |

# 1. Important Build Assumptions

Controller board required The listed electronic modules cannot make decisions by themselves. For working Bluetooth, obstacle, line-following, RF, or servo modes, add one controller board such as Arduino Uno/Nano. Also keep jumper wires, a switch, motor wheels, chassis, and a 5 V regulator available.

The configurations below keep the listed modules as the main robot kit. The controller is treated as the support brain that reads sensors and sends signals to the L293D, servo, HC-05, and RF receiver.

Use a 2S Li-ion pack only with protected cells or a proper holder/BMS; do not short the cells.

Keep all grounds common: battery negative, controller GND, L293D GND, sensor GND, HC-05 GND, and servo GND.

Do not connect HC-05 RX directly to a 5 V TX pin unless your module is explicitly 5 V tolerant; use a voltage divider.

Power the servo from a stable 5 V supply. Avoid pulling heavy servo current from a weak controller 5 V pin.

Test motors on a stand before placing the robot on the floor.

# 2. Common Platform Layout

Build one base rover and swap firmware or sensor connections for each configuration. Mount the BO motors on left and right sides, place the battery pack low and central, keep the breadboard accessible, and mount sensors at the front.

| Subsystem | Recommended placement | Notes |
| --- | --- | --- |
| Drive motors | Left and right sides of chassis | Use differential steering: both forward = forward, left reverse/right forward = turn left. |
| L293D | Breadboard center | Keep motor wires short; L293D can warm up under load. |
| Battery pack | Low center of chassis | 2 x 3.7 V cells in series give 7.4 V nominal for motor supply. |
| Controller | Near breadboard | Use short signal wires to L293D and sensors. |
| Ultrasonic + servo | Front top | Servo sweeps sensor left, center, right for obstacle mode. |
| IR sensors | Front underside | Place 8-15 mm above floor for line tracking, adjust sensitivity screws if present. |
| HC-05 | Top or rear | Keep antenna side clear of metal and motor wires. |
| RF module | Top side | Use a straight wire antenna if the module requires one. |

## Shared Power Architecture

| Power line | Connects to | Purpose |
| --- | --- | --- |
| Battery + | L293D pin 8 / motor supply | Motor voltage from 2S Li-ion pack. |
| 5 V regulated | Controller 5V, L293D pin 16, sensors, HC-05 VCC, servo VCC | Logic and accessory power. |
| GND common | Battery -, controller GND, L293D pins 4/5/12/13, sensors, HC-05, servo | Shared reference for all signals. |
| Switch | Battery + line before distribution | Allows safe power-off while rewiring. |

# 3. L293D Motor Driver Wiring

The L293D controls two DC motors. Each motor uses two direction inputs and one enable pin. Enable pins can be connected to PWM-capable controller pins for speed control.

| L293D pin | Name | Connect to | Function |
| --- | --- | --- | --- |
| 1 | EN1,2 | D5 / PWM | Enable and speed for left motor. |
| 2 | IN1 | D8 | Left motor direction input A. |
| 3 | OUT1 | Left motor terminal 1 | Output to left motor. |
| 4,5 | GND | Common ground | Driver ground. |
| 6 | OUT2 | Left motor terminal 2 | Output to left motor. |
| 7 | IN2 | D9 | Left motor direction input B. |
| 8 | Vcc2 | Battery + motor supply | Motor voltage. |
| 9 | EN3,4 | D6 / PWM | Enable and speed for right motor. |
| 10 | IN3 | D10 | Right motor direction input A. |
| 11 | OUT3 | Right motor terminal 1 | Output to right motor. |
| 12,13 | GND | Common ground | Driver ground. |
| 14 | OUT4 | Right motor terminal 2 | Output to right motor. |
| 15 | IN4 | D11 | Right motor direction input B. |
| 16 | Vcc1 | 5 V logic | L293D logic supply. |

# 4. Configuration A: Bluetooth Remote Rover

Use a phone Bluetooth serial app to send simple commands to the robot. This is the easiest first working build because only the drive system and HC-05 are needed.

| Module | Connection | Notes |
| --- | --- | --- |
| HC-05 TXD | Controller D2 / software serial RX | Receives phone command data. |
| HC-05 RXD | Controller D3 / software serial TX through divider | Protect RX if controller TX is 5 V. |
| HC-05 VCC/GND | 5 V and common GND | Most breakout boards accept 5 V VCC; verify your board. |
| L293D | Use shared motor pin map | D5/D6 can control speed. |

| Phone command | Robot action |
| --- | --- |
| F | Both motors forward. |
| B | Both motors reverse. |
| L | Left turn: left motor reverse/stop, right motor forward. |
| R | Right turn: left motor forward, right motor reverse/stop. |
| S | Stop both motors. |

First pair HC-05 with the phone; common pairing PINs are 1234 or 0000.

Start with 50-60% PWM speed so the robot does not jerk during tests.

If left/right commands feel reversed, swap one motor's two output wires or correct it in firmware.

# 5. Configuration B: Obstacle Avoiding Rover

Mount the ultrasonic sensor on the micro servo. The robot moves forward until an object is detected, then scans left and right before turning toward the clearer side.

| Module | Connection | Notes |
| --- | --- | --- |
| Ultrasonic VCC/GND | 5 V and common GND | Use short wires at the front. |
| Ultrasonic TRIG | D4 | Output pulse from controller. |
| Ultrasonic ECHO | D7 | Input pulse to controller; use divider if required by board. |
| Servo signal | D12 | Use stable 5 V servo power and common GND. |
| L293D | Use shared motor pin map | Same drive base as Bluetooth mode. |

| Step | Control logic |
| --- | --- |
| 1 | Point ultrasonic sensor forward and measure distance. |
| 2 | If distance is greater than 25 cm, move forward. |
| 3 | If distance is less than 25 cm, stop and reverse briefly. |
| 4 | Servo scans left and right; measure both distances. |
| 5 | Turn toward the side with more clearance, then continue forward. |

# 6. Configuration C: IR Line Follower

Place both IR sensors under the front edge, facing the floor. Use black tape on a light floor as the line. The two sensors tell the controller whether the line is left, right, or centered.

| IR sensor state | Robot action |
| --- | --- |
| Left on line, right off line | Turn left slowly. |
| Right on line, left off line | Turn right slowly. |
| Both centered on line | Move forward. |
| Both off line | Stop or search slowly by rotating. |

| Module | Connection | Notes |
| --- | --- | --- |
| Left IR OUT | A0 or D2 | Digital output is easiest; analog output allows threshold tuning. |
| Right IR OUT | A1 or D3 | Keep both sensors at the same height. |
| IR VCC/GND | 5 V and common GND | Adjust onboard trimpot if sensor has one. |
| L293D | Use shared motor pin map | Lower speed improves tracking. |

# 7. Configuration D: Bluetooth Rover with Obstacle Guard

This mode combines manual control with safety. The phone still sends commands, but the ultrasonic sensor prevents forward movement when the path is blocked.

| Command/event | Behavior |
| --- | --- |
| F command and distance clear | Drive forward. |
| F command and distance below 20-25 cm | Stop and ignore forward command until clear. |
| L/R command | Allow turning because turning can clear the obstacle. |
| B command | Allow reverse movement. |
| S command | Stop immediately. |

Why this is useful Manual Bluetooth rovers often crash during first tests. A simple ultrasonic guard makes the robot safer while still keeping full phone control.

# 8. Configuration E: RF Remote Rover

Use the RF module when you want a non-Bluetooth remote experiment. Exact wiring depends on the RF module type. A 433 MHz receiver, for example, normally needs a matching transmitter and a decoding method.

| RF option | How to use it | Support needed |
| --- | --- | --- |
| Simple data receiver | Read data pin on one controller input and map decoded commands to movement. | Compatible transmitter and firmware library. |
| 4-channel RF decoder board | Map channel outputs to forward, reverse, left, right. | Encoder/decoder pair or board with digital outputs. |
| Telemetry experiment | Send sensor state to a paired receiver station. | Second controller/transmitter setup. |

Keep RF and HC-05 modes separate during first tests to avoid pin conflicts.

RF range improves with the correct antenna length for the module frequency.

If the RF module is only a bare transmitter or receiver, it is not a complete remote-control system by itself.

# 9. Configuration F: Multi-Mode Learning Robot

This final configuration uses the same chassis and all listed sensor modules as a reusable learning platform. The controller firmware selects one operating mode at startup or by Bluetooth command.

| Mode | Main modules active | Best classroom/demo use |
| --- | --- | --- |
| Bluetooth RC | HC-05 + L293D + BO motors | Manual control and motor-driver learning. |
| Obstacle avoider | Ultrasonic + servo + L293D | Autonomous decision making. |
| Line follower | 2 IR sensors + L293D | Feedback control and sensor calibration. |
| Guarded Bluetooth | HC-05 + ultrasonic + L293D | Human command with safety override. |
| RF remote | RF module + L293D | Wireless communication comparison. |

# 10. Starter Firmware Structure

The exact sketch depends on the controller board, but the structure is stable. Keep motor functions separate so every mode can reuse the same drive commands.

setup(): initialize motor pins, serial/Bluetooth, sensors, and servo

loop(): read selected mode

if mode == Bluetooth: read command F/B/L/R/S and call motor functions

if mode == Obstacle: measure distance, scan servo, choose turn

if mode == LineFollower: read left/right IR and correct motor speeds

if mode == RF: decode RF channel/data and map to motor commands

motorForward(speed), motorReverse(speed), turnLeft(speed), turnRight(speed), stopMotors()

# 11. Test Plan

| Test | Pass condition | Fix if failed |
| --- | --- | --- |
| Battery voltage check | Pack is charged and polarity is correct. | Recharge safely; correct holder wiring. |
| Common ground check | All modules share GND. | Add missing GND link before signal tests. |
| Motor direction test | Forward command moves both wheels forward. | Swap motor wires or invert logic. |
| HC-05 serial test | Phone command appears in serial monitor or moves robot. | Check baud rate, pairing, TX/RX crossing. |
| Ultrasonic test | Distance reading changes with hand movement. | Check TRIG/ECHO pins and 5 V/GND. |
| Servo scan test | Servo moves smoothly left-center-right. | Use stronger 5 V supply; check common ground. |
| IR sensor test | Sensor state changes over black/white surface. | Adjust trimpot and sensor height. |
| RF test | Receiver output changes when remote transmits. | Confirm matching frequency, encoder, antenna, and library. |

# 12. Troubleshooting

| Symptom | Likely cause | Action |
| --- | --- | --- |
| Robot resets when motors start | Voltage sag or servo/motor current spike. | Use better 5 V regulator, separate servo supply, and common GND. |
| One wheel spins backward | Motor polarity reversed. | Swap that motor's two L293D output wires. |
| Bluetooth pairs but no movement | Wrong baud rate or TX/RX not crossed. | Try 9600 baud, cross HC-05 TX to controller RX and controller TX to HC-05 RX. |
| L293D gets hot | Motor load too high or stalled wheel. | Lift chassis, reduce PWM, check wheel friction. |
| Line follower oscillates | Speed too high or sensors too high. | Lower PWM, reduce sensor height, tune threshold. |
| Ultrasonic reads zero/random | Echo wiring, power noise, or no common ground. | Check pins, shorten wires, add common ground. |
| RF works only very close | No antenna or noisy supply. | Use correct antenna length and keep RF away from motor wires. |

# 13. Recommended Build Order

Assemble chassis, BO motors, wheels, battery holder, and breadboard.

Wire L293D and test motor forward/reverse with the controller.

Build Bluetooth remote mode and verify phone control.

Add ultrasonic sensor fixed forward, then add servo scanning.

Add the two IR sensors and tune line-following mode separately.

Add RF only after motor and sensor behavior is stable.

Combine modes in firmware after every individual mode passes.

Final practical recommendation Build Bluetooth RC first, then obstacle avoiding, then line follower. These three modes prove the drive base, wireless control, distance sensing, servo motion, and IR sensing before attempting a combined multi-mode robot.